Release 1.00-1998

Joos Design

PHOENIX

PROGRAMMER

This Phoneix programmer is based on the version previous released by Odin.

Joos Design have added a better power supply and made some other small changes.

We presume a Phoenix programmer will not be your first build, so a detailed instruction is not supplied.

The PCB drawing shuld be downscaled to about 12 % of the original,

make a test printout and verify you have the right distance for the IC legs.

PART LIST FOR PHOENIX PROGRAMMER

Reference to Component.bmp

POS NO	SPEC.					RS PART NO.

__________________________________________________________

1	Main board

2	Smart Card Connector			197-1197

3	9 Way D connector - socket			446-585

4	Fastening Screws

5	DC Power Connector			486-662

6	Heat Sink - only needed for heavily use		L=19mm W=13mm

7	5V Regulator				853-371

8	Fastening Screw

9	Bridge Rectifier				657-088

10	Resistor 20K ohm				163-763

11	Resistor 220 ohm				149-650

12	Resistor 220 ohm				149-650

13	Resistor 220 ohm				149-650

14	Resistor 2K2 ohm				149-739

15	Resistor 1M  ohm				149-975

16	Not used

17	Crystal 3.579 MHz				226-1500

18	Capacitor 1 uF				200-6167

19	Capacitor 0.1 uF				115-988

20	Capacitor 0.33 uF				116-032

21	Capacitor 100 pF				128-168

22	Capacitor  27 pF				128-763

23	Capacitor  27 pF				128-763

24	Capacitor  33 uF - only needed for heavy use	116-880

25	LED green				590-345

26	LED red					590-339

27	IC 7407N					306-336

27b	IC socket 14 pin				447-314

28	IC MAX232ECPE				225-8510

28b	IC socket 16 pin				447-320

29	IC 74HC04N				217-5550

29b	IC socket 14 pin				447-314

30	Secondary board, on top of smartcard connector.

Mount everything but the secondary board, leds and ICs

Mount top of leds into the secondary board, fasten secondary board to top of smartcard connector

after carefully first having put the led legs through the holes in main board.

Finally , ground yourselves to get rid of any statics and mount the ICs.

---------------------END-----------------------